Always show all work for full credit.

- Determine the minimal expression minimal (the Karnaugh map sense) for Y in:

| A | B | C | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

- Determine the minimal expression minimal (the Karnaugh map sense) for Y in:

| A | B | C | D | Y |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

- Determine the minimal expression minimal (the Karnaugh map sense) for Y in:

| A | B | C | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

- Determine the minimal expression minimal (the Karnaugh map sense) for Y in:

| A | B | C | D | Y |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

- Problems 1-4 used the same functions as problems 2-5 on Homework 2A.  In what way are the equations optimal or minimal compared to the equations in Homework 2A?

- Determine a minimal (the Karnaugh map sense) form of:

Y=BC+ABC+BC

- Determine a minimal (the Karnaugh map sense) form of:

Y=A+AB+AB+A+B

- Give the minimized (the Karnaugh Map sense) Boolean equation for the function performed by the multiplexors below. Note that B is connected to the inputs to the leftmost multiplexor.  The “T” shape on the right multiplexor’s input indicates a voltage source (that is, a logic 1) and the triangle on the left multiplexor’s input represents a ground (that is, a logic 0):

9. Use an 8-to-1 Multiplexor to implement the same function from the previous problem.

10. Assuming the propagation delays are:

| Gate | tpd (ps) |
| --- | --- |
| NOT | 15 |
| 2-input NAND | 20 |
| 3-input NAND | 30 |
| 2-input NOR | 30 |
| 3-input NOR | 45 |
| 2-input AND | 30 |
| 3-input AND | 40 |
| 2-input OR | 40 |
| 3-input OR | 55 |
| 2-input XOR | 60 |

Determine the propagation delay of:

11.1 Complete the full truth table for the multiplexor with enable (described on assignment page):

  - Find the full, canonical sum-of-products equation for it.

  - (JLS Submission)

11.4 What is the Propagation delay in JLS (using default delays)?  (You can right-click on gates and select “Change Timing” to examine their default timing values, but do not change them).

  - Give the minimized (the Karnaugh Map sense) Boolean equation for the function.

  - (JLS Submission)

  - What is the Propagation delay in JLS (using default delays) of the minimized circuit?

  - Compare and contrast the minimal (K-Map) version with the full Sum-of-Products version.